BLADDER TANK ORIZONTAL PENTRU SPUMA CU MEMBRANA INTERNA

HORIZONTAL FOAM BLADDER TANK INTERNAL MEMBRANE

Mod. FI-OPI

| Caracteristici tehnice | Technical construction data |
| --- | --- |
| - Norma: 2014/68/UE (Ex. PED 97/23/CE), ASME VIII Div.1 la cerere | - Construction code: 2014/68/UE (Ex. PED 97/23/CE), ASME VIII Div.1 on request |
| - Presiune de design: 12 bar | - Design pressure: 12 bar |
| - Presiune maxima de lucru: 12 bar | - Max working pressare: 12 bar |
| - Presiune de testare: 18 bar | - Test pressure: 18 bar |
| - Temperatura de design: -10° + 50°C | - Design temperature: -10° +50°C |

Data Sheet N°52

T

ABELLA DIMENSIONALE

DIMENSIONAL TABLE

| Caracteristici tehnice constructive | Technical construction data |
| --- | --- |
| Material | Materials |
| - Capac : P355 NH | - Sheel and head: P355 NH |
| - Flanse: ASTM A 350 LF2 (UNI o ANSI) | - Flange: ASTM A 350 LF2 (UNI o ANSI) |
| - Teava: ASTM A 333 Gr.6 | - Pipe: ASTM A 333 Gr.6 |
| - Valva apa si spuma: ASTM A 105 cu sfera in AISI 304 | - Water and foam ball valve: ASTM A 105 with ball in AISI 304 |
| - Valva de drenaj si scurgere: Alama | - Drain and intercept valve: Brass |
| - Manometru: AISI 304 | - Pressare gauge: AISI 304 |
| - Duza apa si spuma: Otel inoxidabil | - Water and foam nozzles: stainless steel |
| - Membrana: Hypalon | - Bladder: Hypalon |
| Ciclu vopsire | Painting cicle |
| - Preparare: sablare SA 2,5 | - Surface preparation: sand blasting SA 2,5 |
| - Prima mana: Epoxivinilic 60 Microns | - Primer: Epoxivinilic 60 Microns |
| - Final: Email poliuretanic 60 Microns rosu RAL 3000 | - Finish: Enamel poliurethanic 60 Microns red RAL 3000 |

La cerere rezervoarele se pot furniza cu diferite valori de presiune

On request tanks can be supplied with different pressure values

TABEL DIMENSIUNI

DIMENSIONAL TABLE

| MODEL MODEL | DIMENSIUNE | – DIMENSION |  |  |  |  | GREUTATE Kg. EMPTY WEIGHT Kg. | CAPACITATE Lt. CAPACITY Lt. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F |  |  |
| FI-OPI 10 | 1000 | 1670 | 1605 | 900 | 700 | 750 | 610 | 1.000 |
| FI-OPI 15 | 1000 | 2260 | 1600 | 900 | 1100 | 750 | 710 | 1.500 |
| FI-OPI 20 | 1100 | 2525 | 1705 | 1000 | 1250 | 800 | 960 | 2.000 |
| FI-OPI 25 | 1170 | 2760 | 1775 | 1000 | 1350 | 800 | 1095 | 2.500 |
| FI-OPI 30 | 1270 | 2840 | 1875 | 1150 | 1400 | 860 | 1180 | 3.000 |
| FI-OPI 35 | 1360 | 2880 | 1965 | 1200 | 1425 | 900 | 1500 | 3.500 |
| FI-OPI 40 | 1450 | 2935 | 2060 | 1250 | 1450 | 950 | 1570 | 4.000 |
| FI-OPI 45 | 1450 | 3200 | 2060 | 1250 | 1450 | 950 | 1645 | 4.500 |
| FI-OPI 50 | 1600 | 3010 | 2215 | 1400 | 1500 | 1050 | 1745 | 5.000 |
| FI-OPI 55 | 1600 | 3225 | 2215 | 1400 | 1500 | 1050 | 1815 | 5.500 |
| FI-OPI 60 | 1750 | 3095 | 2365 | 1500 | 1500 | 1150 | 1920 | 6.000 |
| FI-OPI 65 | 1750 | 3210 | 2365 | 1500 | 1500 | 1150 | 1960 | 6.500 |
| FI-OPI 70 | 1750 | 3445 | 2365 | 1500 | 1900 | 1150 | 2100 | 7.000 |
| FI-OPI 75 | 1750 | 3655 | 2365 | 1500 | 1900 | 1150 | 2350 | 7.500 |
| FI-OPI 80 | 1960 | 3225 | 2575 | 1700 | 1500 | 1250 | 2230 | 8.000 |
| FI-OPI 85 | 1960 | 3375 | 2575 | 1700 | 1650 | 1250 | 2325 | 8.500 |
| FI-OPI 90 | 2100 | 3305 | 2710 | 1850 | 1500 | 1300 | 2850 | 9.000 |
| FI-OPI 100 | 2200 | 3365 | 2810 | 1950 | 1500 | 1350 | 3035 | 10.000 |
| FI-OPI 110 | 2200 | 3495 | 2810 | 1950 | 1500 | 1350 | 3120 | 11.000 |
| FI-OPI 120 | 2200 | 3765 | 2810 | 1950 | 1900 | 1350 | 3300 | 12.000 |
| FI-OPI 130 | 2200 | 4035 | 2870 | 1950 | 2150 | 1350 | 3190 | 13.000 |
| FI-OPI 140 | 2200 | 4365 | 2870 | 1950 | 2480 | 1350 | 3370 | 14.000 |
| FI-OPI 150 | 2200 | 4615 | 2870 | 1950 | 2730 | 1350 | 3500 | 15.000 |